Title: 
Electrowinning of Ferromanganese by Molten Oxide Electrolysis (MOE), with Consumable Iron-Cathode

Abstract:

Global warming is one of the biggest challenges ever hitting humanity. It is a result of decades of emitting more greenhouse gases, such as CO2, than the environment could capture. The metallurgical industry accounts for a major share of these emissions, as many production processes are directly dependent on carbon as a reductant. Ferromanganese, a critical alloy in steelmaking, is conventionally produced via carbothermic reduction in submerged arc furnaces, resulting in up to 2 metric tones of CO2 per metric tone of ferromanganese. To reduce the environmental impact of ferromanganese production, alternative carbon-free processes are needed. This project investigates the feasibility of producing ferromanganese through molten oxide electrolysis (MOE), a process that will replace carbon with electrical energy as the reducing agent. MOE have been demonstrated to work for metals such as iron, but its application to manganese slags is not studied sufficiently. In this study, laboratory scale electrolysis experiments were conducted at 1300 oC in inert argon atmosphere, using synthetic oxide melts designed to approximate industrial slag compositions. Electrochemical parameters and phase compositions will be characterized using SEM-EDS and numerical calculations. Although the experimental work is ongoing, I have this far gotten promising results indicating that MOE could be a feasible process for ferromanganese production.  Challenges have been the corrosivity of the melt at these high temperatures, making it hard to find suitable materials for crucible and electrodes. The work will mainly concentrate from now on finding the best melt composition for MOE. If successful, this research may contribute to developing sustainable metallurgical processes and reducing the carbon footprint of steel production. It may also support further research on carbon-free production of other metals.

One-liner:

By using molten oxide electrolysis, this project investigates a carbon free production process for ferromanganese to mitigate CO2 emissions in the metallurgical industry by replacing carbon with electrical energy, aiming to demonstrate technical feasibility and identify key process parameters for stable high yield production.

Keywords:

- MOE,

- Electrochemistry,

- Carbon-free,

- Ferromanganese,

- Metallurgy

Reflection:

For the abstract I selected a 300-word limit since this matched the requirement in the assignment and also seems to be quite similar to what is expected from me on the project report. I chose to go a little bit under the limit since I wanted to keep it clear and avoid unnecessary details. Since I’m not yet finished with my experiments I chose to structure the abstract in a tentative way. Focusing mainly on the problem and choice of method. This follows the advice in the assignment.

I followed the mini-IMRAD template from the lectures, including the backround, purpose, method, and anticipated outcomes or contributions. For the one-liner I chose to follow the template given. This helped makit clear and concise, telling what the project is about. I think the one-liner was the hardest to write since I wanted to include so much, but in the end it helped me to clarify the projects essence in just one sentence.

The most challenging part of writing the abstract was to find a balance in how technical it should be. And how much background to include. The audience is assumed to be accademics with some general chemistry or metallurgy background. I tried to avoid the most specific details but also tried to include the most important aspects of the project and its experiments. The template given from the lectures made the process of writing the abstract much easier since I got some pointers to how to write it.